# MAC Address Assignment – Due Dec 1

Do this assignment in a (.txt), rich text (.rtf), or MS Word (.doc) document. Go to 5 different computers or devices to get the physical (MAC) address for its Ethernet or wireless interface. Choose machines that no one else in class will use (I do not want the same machines on any of your papers, so try to find one that no one else will go to for their 5 machines). Do not use more than one computer from the same lab. Include the following information in your paper for each of the 5 interfaces:

- MAC Address - Report this in hexadecimal XX-XX-XX-XX-XX-XX format.

- Machine - Give the location and description of the machine. I say "machine" since devices other than computers have interfaces such as printers or routers.

- Lookup Method - Explain briefly how you got the MAC address? That is, what command did you enter on this computer or some remote computer (e.g., remotely to discover the MAC address of a printer's interface).

- Vendor - Give the name for the manufacturer of the NIC for each of the following ways (it might vary):

  - Search the first half of the MAC address (format is XX-XX-XX) on-line at: http://standards.ieee.org/regauth/oui/index.shtml

  - Search the first half of the MAC address (format is XX:XX:XX) on-line at: http://www.coffer.com/mac_find

  - If possible (see Further Instructions) write down the vendor name that the computer itself reports to you. If you cannot figure out how to get this, just say Cannot Determine.

## Further Instructions/Hints

This assignment is a modified version of Challenge Exercise 4.3 (page 224-25) so you might get some ideas from that. To get the MAC address of an interface for the machine that you are sitting at or logged on to, enter one of the following commands:

ipconfig /all     - if on Windows XP
ifconfig -a     - if on Unix or MacOS(10+)

To get the MAC address of some other device on the same LAN you can ping it and then look at the ARP table soon thereafter before the table drops old MAC address information. This may not work from your dorm or office PC if it is connected to a wall port wired to a switch (or router), because that switch is the only other device on your segment. Therefore, you might try logging into trinity or neo and pinging some server, printer, or device from there. For example, if you wanted the MAC address of foo.graceland.edu do the following:

ping foo     - enter CTRL-C to kill it at some point
arp -a     	- do this soon thereafter to get whichever MAC address you are interested in